Monitoring Data for 1st and 2nd February possession works

|  | Noise Monitoring Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location |  | Date | Time | Measured (LAeq 15 min) | Predicted noise level | Comments |
| 32 Dwyer Street, North Gosford |  | 01/02/14 | 1310 | 57.5 | 72 | Excavation of footings and tamping works. |
| 46 Hills St, North Gosford |  | 01/02/14 | 1430 | 45 | 47.6 | Excavation of footings and tamping works. Concrete truck with delivery. |
| 24 Campbell street, North Gosford |  | 01/02/14 | 1455 | 60 | 69.3 | Excavation of footings and concrete truck accessing the corridor. |
| 192 Showground Road, Narara. |  | 01/02/14 | 1545 | 40.6 | 59.5 | Excavation of footings and drilling at the Wyoming Bridge. |
| 46 Hills St, North Gosford |  | 01/02/14 | 1625 | 45.5 | 47.6 | Excavation of footings and tamping works. |
| 24 Campbell street, North Gosford |  | 01/02/14 | 1648 | 57.5 | 69.3 | Excavation of footings and rock breaking works. |
| 24 Campbell street, North Gosford |  | 01/02/14 | 1710 | 57.0 | 69.3 | Excavation of footings, tamping, and rock breaking works. |
| 24 Campbell street, North Gosford |  | 02/02/14 | 0205 | 55.1 | 68.5 | Installation of overhead electrical structure. 1 crane and 2 lighting towers operating on the down. |
| 24 Campbell street, North Gosford |  | 02/02/14 | 0245 | 57.2 | 68.5 | Installation of overhead electrical structure. Drilling into structure with the use of hand tools and generator.1 crane and 2 lighting towers operating on the down. |
| 24 Campbell street, North Gosford |  | 02/02/14 | 0330 | 61.3 | 68.5 | Installation of overhead electrical structure. 2 EWPs operating positing structure. 1 crane and 2 lighting towers operating on the down and up. |
| 25 Campbell street, North Gosford |  | 02/02/14 | 0423 | 42.2 | 63.5 | Installation of overhead electrical structure. 2 EWPs, 1 crane and 2 lighting towers operating on the down and up. |
| 32 Dwyer St, North Gosford |  | 02/02/14 | 1010 | 53.9 | 72 | Excavation of footings. 3 excavators operating, 2 10t dump trucks and light vehicles operating on the down. |
| 24 Campbell street, North Gosford |  | 02/02/14 | 1058 | 66.4 | 69.3 | Service location and excavation. Vacuum truck, 2 excavators and 1 10t dump truck operating on the down. |

NOTES

Monitoring results showed that actual noise impacts were below predicted levels. Affected sensitive receivers were notified of the upcoming works via letter box drops and where required were offered alternate accommodation in accordance with the projects Noise and Vibration Management Plan and the Community Liaison Plan. Monitoring for the footings at Ch 81.595 and Ch 82.220 was done at 25m and 60m respectively and a calculated impact produced due to high level of non-construction related noise at receiver.