Noise Monitoring Data for 23th and 24th November 2013 Possession Works

|  | Noise Monitoring Results |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Location |  | Date | Time | Measured (LAeq 15 min) | Predicted noise level (P139-GPLNVIS-r1.2) | Comments |
| Glennie St |  | 24/11/13 | 03:38 | 47.8 | 46.2 | Signalling works – lighting tower at gate A3. |
| 10 Akora Road |  | 23/11/13 | 08:10 | 52.1 | 80.3 | Monitoring of tree loppers and excavator operating on the Down track. |
| 10 Akora Rd |  | 23/11/13 | 14:15 | 79.0 | 80.3 | Clearing and grubbing works adjacent to Akora Rd |
| 522 – 528 Pacific Hwy |  | 24/11/13 | 09:02 | 56.0 | 57.1 | Monitoring of crane and sheet piling |
| 522 – 528 Pacific Hwy |  | 24/11/13 | 09:22 | 56.9 | 57.1 | Monitoring of crane and sheet piling |
| 522 – 528 Pacific Hwy |  | 24/11/13 | 09:43 | 59.7 | 57.1 | Monitoring of crane and sheet piling |
| 522 – 528 Pacific Hwy |  | 24/11/13 | 14:30 | 56 | 57.1 | Monitoring of crane and sheet piling |
| 32 Dwyer St |  | 23/11/13 | 08:16 | 63.2 | 50.1 | Monitoring of 2 to 3 Excavators working simultaneously – open cut ULX |
| 32 Dwyer St |  | 23/11/13 | 14:10 | 55.9 | 50.1 | Monitoring of 2 to 3 Excavators working simultaneously – open cut ULX |
| 31 Dwyer St |  | 24/11/13 | 16:46 | 52.1 | 50.1 | Monitoring of 2 to 3 Excavators working simultaneously – open cut ULX |
| 31 Dwyer St |  | 24/11/13 | 17:10 | 54.8 | 50.1 | Monitoring of 2 to 3 Excavators working simultaneously – open cut ULX |
| 31 Dwyer St |  | 23/11/13 | 20:35 | 53.5 | 86.9 | Monitoring of 80t crane setting up on Down with lighting tower and dismantling overhead structures. |
| 31 Dwyer St |  | 23/11/13 | 23:19 | 55.0 | 86.9 | Monitoring of removal of redundant infrastructure and a crane operating. |
| 31 Dwyer St |  | 24/11/13 | 03:00 | 53.5 | 86.9 | Monitoring of crane at idle and lighting tower. |

NOTES

Monitoring for the sheet piling was done at 20m from the piling rig and a calculated impact produced due to high level of non-construction related noise at receiver. Two excedances of predicted noise levels were recorded. Details follow; - Glennie Street, minor exceedance of predicted noise levels recorded. Results were below noise management levels for commercial receivers. - 522 – 528 Pacific Hwy, exceedance of predicted noise levels recorded. The noise mitigation measures implemented for the receiver were still in compliance with the Project Noise and Vibration plan and TfNSW CNS